DESIGN AND DEVELOPMENT OF

ELECTRIC BIKE

[Project Report]

ABSTRACT

The increasing demand for sustainable transportation, rising fuel costs and growing environmental concerns have accelerated the development of electric vehicles as an efficient alternative to conventional fuel-powered transportation. This project presents the design and development of an Electric Bike that provides an economical, environment friendly and energy-efficient solution for short and medium-distance travel purpose. The proposed system integrates a Brushless DC (BLDC) motor powered by a lithium-ion battery pack, offering high efficiency, low maintenance and reliable performance.

The electric bike is designed with a lightweight yet durable frame that accommodates the battery pack, motor, controller, and other essential electrical components while maintaining proper weight distribution and rider comfort. A high-performance BLDC motor is selected to deliver smooth acceleration and a maximum speed suitable for urban transportation. The lithium-ion battery provides sufficient energy storage to achieve an extended driving range on a single charge while reducing charging frequency and operating costs.

The vehicle incorporates an LCD display that provides real-time information, including speed, battery level, trip distance and other operational parameters, enabling the rider to monitor the system effectively. For enhanced safety, the bike is equipped with Brembo hydraulic disc brakes, providing reliable braking performance under various riding conditions. The suspension system consists of hydraulic telescopic front forks and adjustable rear shock absorbers, ensuring improved ride comfort, vehicle stability and effective vibration damping on uneven road surfaces.

The design process includes component selection, system modeling, mechanical layout, electrical integration and performance analysis to ensure efficient operation and structural reliability. The completed design demonstrates the practical application of electric vehicle technology by combining efficiency, safety, durability, and rider comfort within a compact two-wheeled platform. This project highlights the potential of electric bikes as a sustainable mode of transportation capable of reducing fuel consumption, lowering greenhouse gas emissions and minimizing maintenance costs while meeting the daily mobility requirements of modern society.

TABLE OF CONTENTS:

| Sr No. | TOPICS | Page No. |
| --- | --- | --- |
| 1. | OBJECTIVES | 4 |
| 2. | PROBLEMS | 5-6 |
| 3. | PROPOSED SOLUTIONS | 7-10 |
| 4. | COMPONENTS AND MATERIALS USED WITH SPECIFICATIONS | 10-16 |
| 5. | DESIGN CALCULATIONS | 17-23 |
| 6. | FABRICATION PROCESS | 23 |
| 7. | ASSEMBLY PROCESS | 23 |
| 8. | WORKING PRINCIPLE | 24 |
| 9. | COST ESTIMATION | 24-25 |
| 10. | LIMITATIONS OF THE PROPOSED MODEL | 25-26 |
| 11. | CONCLUSION | 26 |
| 12. | CAD MODEL AND ASSEMBLY | 27-28 |
| 13. | REFERENCES | 29 |

- Objectives

- Design and develop an electric bike capable of safely carrying two adult riders with a strong and durable frame.

- Select a 2 kW BLDC motor and a 60 V lithium-ion battery to achieve a maximum speed of approximately 50 km/h with efficient power delivery.

- Integrate an efficient electrical drive system comprising the BLDC motor, motor controller, battery management system (BMS), and throttle for smooth and reliable operation.

- Equip the bike with a multifunction LCD display to provide real-time information such as speed, battery level, and trip distance.

- Enhance rider safety by incorporating Brembo hydraulic disc brakes and a hydraulic suspension system for superior braking performance and riding comfort.

- Evaluate the bike's performance in terms of speed, load-carrying capacity, battery range, structural stability, and overall efficiency to ensure compliance with the design objectives.

- PROBLEMS

The transportation sector is one of the largest contributors to fuel consumption and environmental pollution. Conventional fuel-powered motorcycles, although widely used, suffer from several drawbacks related to efficiency, operating cost, and environmental impact. Likewise, many existing electric bikes available in the market utilize low-cost components that compromise performance, reliability, and rider comfort. These limitations motivated the development of the proposed electric bike using an optimized combination of modern electrical and mechanical components.

Problems Associated with Fuel-Based Motorcycles

1. High Fuel Consumption

Conventional motorcycles rely on petrol or diesel, whose prices continue to increase over time. Daily commuting results in significant fuel expenses, making transportation costly for users.

2. Environmental Pollution

Internal combustion engines emit harmful gases such as carbon dioxide (CO₂), carbon monoxide (CO), nitrogen oxides (NOₓ), and unburned hydrocarbons, contributing to global warming and air pollution.

3. High Maintenance Requirements

Fuel-powered engines consist of numerous moving parts that require regular servicing, including engine oil replacement, spark plug maintenance, clutch adjustments, air filter cleaning, and exhaust system servicing.

4. Low Energy Efficiency

A large portion of the fuel's energy is lost as heat through the engine and exhaust system, resulting in comparatively lower overall efficiency than electric propulsion systems.

5. Noise and Vibration

Combustion engines generate considerable noise and vibration, reducing rider comfort and contributing to noise pollution in urban environments.

6. Dependence on Fossil Fuels

Fuel-powered motorcycles rely entirely on non-renewable fossil fuels, making them unsustainable as fuel resources continue to diminish.

Problems Found in Many Existing Electric Bikes

Although electric bikes eliminate many of the disadvantages of fuel-powered motorcycles, several commercially available low-cost electric bikes still exhibit notable limitations.

1. Use of Brushed DC Motors

Many entry-level electric bikes employ brushed DC motors due to their lower initial cost. However, these motors have brushes that wear over time, require frequent maintenance, generate heat, and operate with lower efficiency.

2. Lead-Acid Batteries

Some electric bikes continue to use lead-acid batteries because of their low purchase price. These batteries are heavy, have lower energy density, longer charging times, shorter service life, and provide limited travel range.

3. Basic Mechanical Braking Systems

Low-cost electric bikes often use mechanical drum or cable-operated disc brakes, which offer lower braking performance and require frequent adjustment compared to hydraulic braking systems.

4. Conventional Suspension Systems

Basic spring suspension provides limited vibration damping and reduced comfort, especially when traveling on uneven roads or carrying additional load.

5. Lack of Smart Monitoring

Many economical electric bikes lack digital displays, preventing riders from monitoring battery percentage, speed, trip distance, and system status accurately.

6. Reduced Performance

The combination of low-capacity batteries and lower-efficiency motors often results in reduced acceleration, lower maximum speed, and shorter travel distance per charge.

- PROPOSED SOLUTION

To overcome the limitations of conventional motorcycles and existing electric bikes, the proposed electric bike incorporates a carefully selected combination of electrical and mechanical components that improve efficiency, safety, reliability, and rider comfort.

1. BLDC Motor Instead of a Brushed DC Motor

A Brushless DC (BLDC) motor is selected as the propulsion system due to its superior performance and efficiency.

| Brushed DC Motor | BLDC Motor (Selected) |
| --- | --- |
| Brushes wear out with time | Brushless construction eliminates wear |
| Requires regular maintenance | Virtually maintenance-free |
| Lower efficiency | Higher efficiency (typically 85–95%) |
| Generates more heat | Better thermal performance |
| Shorter lifespan | Longer operational life |
| Lower torque | Higher torque-to-weight ratio |

Reason for Selection:
The BLDC motor offers higher efficiency, smoother operation, reduced maintenance, and longer service life, making it ideal for electric vehicles.

2. Lithium-Ion Battery Instead of Lead-Acid Battery

The proposed bike uses a Lithium-Ion battery pack rather than a lead-acid battery.

| Lead-Acid Battery | Lithium-Ion Battery (Selected) |
| --- | --- |
| Heavy | Lightweight |
| Lower energy density | Higher energy density |
| Shorter lifespan | Longer cycle life |
| Slow charging | Faster charging |
| Higher maintenance | Maintenance-free |
| Lower travel range | Greater travel range |

Reason for Selection:
Lithium-ion batteries significantly reduce vehicle weight while increasing range, charging efficiency, and overall battery life.

3. Hydraulic Brembo Disc Brakes Instead of Mechanical Brakes

For the braking system, Brembo hydraulic disc brakes are employed.

| Mechanical Brakes | Brembo Hydraulic Disc Brakes (Selected) |
| --- | --- |
| Higher braking effort | Lower lever effort |
| Less precise braking | Excellent braking control |
| Requires cable adjustment | Self-adjusting hydraulic system |
| Reduced stopping performance | Superior stopping power |

Reason for Selection:
Hydraulic disc brakes provide shorter stopping distances, improved safety, and consistent braking performance in both dry and wet conditions.

4. Hydraulic Suspension Instead of Conventional Spring Suspension

The proposed bike utilizes hydraulic telescopic front forks and adjustable rear hydraulic shock absorbers.

| Conventional Suspension | Hydraulic Suspension (Selected) |
| --- | --- |
| Limited damping | Excellent vibration damping |
| Reduced comfort | Improved ride comfort |
| Less stability | Better handling and stability |
| Lower adaptability | Adjustable for varying road conditions |

Reason for Selection:
Hydraulic suspension enhances rider comfort, minimizes vibration, and improves vehicle stability across different terrains.

5. LCD Display Instead of Analog Indicators

The bike is equipped with a modern LCD display.

| Analog Meter | LCD Display (Selected) |
| --- | --- |
| Limited information | Multiple real-time parameters |
| Difficult to monitor battery | Accurate battery monitoring |
| No trip information | Trip and odometer functions |
| No fault indication | Diagnostic and warning indicators |

Reason for Selection:
The LCD display provides comprehensive real-time information, enabling better rider awareness and efficient battery management.

- Materials and Components Used and Specifications

4.1. Frame and Chassis

| Parameter | Specification |
| --- | --- |
| Material | Mild Steel (AISI 1018 / ASTM A36) |
| Manufacturing Process | MIG Welding |
| Shape | Rectangular & Circular Hollow Sections |
| Wall Thickness | 2.5–3 mm |
| Factor of Safety | ≥ 2 |

Reason for Selection:

- High structural strength

- Good weldability

- Economical

- Easily available

- Suitable for carrying two adults

4.2. Electric Motor

| Parameter | Specification |
| --- | --- |
| Type | Brushless DC (BLDC) Hub Motor |
| Rated Power | 2000 W (2 kW) |
| Rated Voltage | 60 V |
| Peak Torque | 70–80 Nm |
| Rated Speed | 650–700 RPM |
| Maximum Vehicle Speed | ≈ 50 km/h |
| Cooling | Air Cooled |
| Efficiency | 88–92% |

Reason for Selection:

- High efficiency

- Maintenance-free

- High starting torque

- Long service life

4.3. Battery Pack

| Parameter | Specification |
| --- | --- |
| Type | Lithium-Ion |
| Voltage | 60 V |
| Capacity | 30 Ah |
| Energy | 1800 Wh (1.8 kWh) |
| Cell Type | 18650 or 21700 Lithium-Ion Cells |
| Battery Management System | Integrated BMS |
| Charging Time | 5–6 Hours |

Estimated Range

Approximately 60–70 km per full charge

4.4. Motor Controller

| Parameter | Specification |
| --- | --- |
| Type | BLDC Controller |
| Rated Voltage | 60 V |
| Current Rating | 40–45 A |
| Control Method | PWM |
| Protection | Over-current, Over-voltage, Under-voltage, Short Circuit |

4.5. Battery Management System (BMS)

| Parameter | Specification |
| --- | --- |
| Battery Compatibility | 60 V Lithium-Ion |
| Continuous Current | 45 A |
| Functions | Cell Balancing, Over-charge Protection, Over-discharge Protection, Short Circuit Protection |

4.6. Throttle

| Parameter | Specification |
| --- | --- |
| Type | Hall Effect Twist Throttle |
| Operating Voltage | 5 V |
| Output Signal | 1–4 V |

4.7. Display

| Parameter | Specification |
| --- | --- |
| Type | LCD Display |
| Display Functions | Speed, Battery Percentage, Odometer, Trip Meter, Voltage, Error Codes |
| Backlight | LED |

4.8. Braking System

| Parameter | Specification |
| --- | --- |
| Brand | Brembo |
| Type | Hydraulic Disc Brakes |
| Front Disc | 220 mm |
| Rear Disc | 220 mm |
| Brake Fluid | DOT 4 |

4.9. Front Suspension

| Parameter | Specification |
| --- | --- |
| Type | Hydraulic Telescopic Fork |
| Travel | 120 mm |
| Material | High Strength Steel |

4.10. Rear Suspension

| Parameter | Specification |
| --- | --- |
| Type | Adjustable Hydraulic Mono Shock |
| Travel | 100 mm |
| Spring Type | Coil Spring |

4.11. Wheels

| Parameter | Specification |
| --- | --- |
| Material | Cast Aluminum Alloy |
| Diameter | 17 inches |
| Rim Type | Alloy |

4.12. Tires

| Parameter | Specification |
| --- | --- |
| Front Tire | 90/90–17 |
| Rear Tire | 100/80–17 |
| Type | Tubeless Radial |

4.13. Handlebar

| Parameter | Specification |
| --- | --- |
| Material | Aluminum Alloy |
| Width | 400 mm |

4.14. Seat

| Parameter | Specification |
| --- | --- |
| Type | Dual Rider Seat |
| Cushion Material | High Density PU Foam |
| Cover Material | Waterproof Leatherette |

4.15. Lighting System

| Parameter | Specification |
| --- | --- |
| Headlight | LED |
| Tail Light | LED |
| Turn Indicators | LED |
| Brake Light | LED |

4.16. Charger

| Parameter | Specification |
| --- | --- |
| Type | Smart Lithium-Ion Charger |
| Input | 220–240 VAC |
| Output | 67.2 V DC |
| Charging Current | 5 A |

4.17. Wiring

| Parameter | Specification |
| --- | --- |
| Material | Copper |
| Insulation | PVC |
| Wire Gauge | 10–18 AWG (depending on load) |

4.18. Fasteners

| Parameter | Specification |
| --- | --- |
| Material | Stainless Steel (Grade 304) |
| Bolt Grade | 8.8 High Tensile |

4.19. Bearings

| Parameter | Specification |
| --- | --- |
| Type | Deep Groove Ball Bearings |
| Material | Chrome Steel |

20. Side Stand

| Parameter | Specification |
| --- | --- |
| Material | Mild Steel |
| Finish | Powder Coated |

- Design Calculations

5.1 Design Parameters

The following design parameters are considered throughout the analysis.

| PARAMETER | SYMBOL | VALUE |
| --- | --- | --- |
| Vehicle Weight | Wb | 60 kg |
| Rider Weight (2 persons) | Wr | 140 kg |
| Total Weight | M | 200kg |
| Maximum Speed | V | 50 km/h (13.89 ms) |
| Wheel Diameter | D | 0.50 m |
| Wheel Radius | R | 0.25 m |
| Battery Voltage | Vb | 60 V |
| Battery Capacity | C | 30 Ah |
| Motor Efficiency | Η | 90 % |

5.2 Total Vehicle Load

The total design load is

m=Wb+Wr
m=60+140
m=200  kg

This load is used for selecting the motor, frame, suspension and braking system.

5.3 Wheel Rotational Speed

Wheel circumference

C=πD
C=3.1416×0.50
C=1.57  m

Vehicle speed

V=50  km/h
Convert to m/s

V=50×10003600
V=13.89  m/s

Wheel RPM

RPM=V×60C
RPM=13.89×601.57
RPM≈531

Therefore, the selected BLDC motor should provide approximately 530–600 RPM.

5.4 Required Traction Force

Rolling resistance coefficient

Crr=0.015

Normal force

N=mg
N=200×9.81
N=1962  N

Rolling resistance

Fr=CrrN
Fr=0.015×1962
Fr=29.4  N

Considering aerodynamic drag and acceleration,

Total tractive force    
Ft≈120  N

5.5 Motor Power Requirement

Power is

P=F×V
P=120×13.89
P=1667  W

Including transmission losses and safety factor

Prequired=1667×1.2
Prequired≈2000  W

Therefore, Selected Motor :  2 kW BLDC Hub Motor

5.6 Motor Torque Calculation

Power equation

P=2πNT60

Rearranging

T=60P2πN

Where

P = 2000 W

N = 550 RPM

T=60×20002×3.1416×550
T=34.7  Nm

A hub motor with peak torque of 70–80 Nm is therefore suitable.

5.7 Battery Capacity

Battery Energy

E=V×Ah
E=60×30
E=1800  Wh

or

1.8  kWh

5.8 Battery Current

Motor Current

I=PV
I=200060
I=33.3  A

A controller rated

40–45 A

is selected.

5.9 Estimated Driving Range

Typical energy consumption

30  Wh/km

Range

Range=Battery EnergyEnergy Consumption
Range=180030
Range=60  km

Estimated driving range

≈ 60 km per full charge

5.10 Battery Charging Time

Charging current

5 A

Battery Capacity

30 Ah

Charging time

t=CapacityCharging Current
t=305
t=6  hours

Considering charging losses

Charging Time≈6-7  hours

5.11 Braking Force

Desired deceleration

a=5  m/s2

Braking force

F=ma
F=200×5
F=1000  N

Hence,

Brembo hydraulic disc brakes are selected to safely generate the required braking force.

5.12 Suspension Load

Assuming

Front Load 40 %

Rear Load 60 %

Front Suspension:

0.40×200
80  kg

Rear Suspension:

0.60×200
120  kg

Thus,

Hydraulic telescopic front forks and adjustable rear hydraulic shock absorbers are selected.

5.13 Frame Design Load

Maximum Design Load

200  kg

Factor of Safety

FOS=2

Design Load

200×2
400  kg

Therefore, the chassis should safely withstand an equivalent static load of 400 kg, justifying the use of AISI 1018/1020 mild steel tubing with adequate wall thickness.

6.1 Fabrication Process

The fabrication of the electric bike began with the construction of the frame using AISI 1018 mild steel tubes which were cut to the required dimensions and joined using MIG welding. After welding, the frame will be inspected for dimensional accuracy and structural integrity before being cleaned and finished. Mounting points for the BLDC motor, lithium-ion battery pack, controller, suspension system33 and braking components were incorporated into the frame to ensure proper alignment and secure installation. The fabricated frame was then prepared for final assembly.

6.2 Assembly Process

The assembly process involves mounting the 2 kW BLDC hub motor to the rear wheel, followed by the installation of the 60 V lithium-ion battery pack, motor controller, and Battery Management System (BMS). The front telescopic forks, rear hydraulic shock absorbers, Brembo hydraulic disc brakes, wheels, handlebars and seat were then assembled onto the frame. The electrical wiring, throttle, LCD display, lighting system and charging port are connected according to the circuit design. Finally, all mechanical and electrical connections were inspected, tightened, and tested to ensure the electric bike operated safely and efficiently.

Working Principle:

The proposed electric bike operates by converting electrical energy stored in the lithium-ion battery into mechanical energy through a Brushless DC (BLDC) motor. When the rider turns the ignition switch ON and applies the throttle, a signal is sent to the motor controller. The controller regulates the power supplied from the battery to the BLDC motor according to the throttle input, allowing smooth acceleration and speed control. The rotating motor drives the rear wheel, propelling the bike forward.

During operation, the Battery Management System (BMS) continuously monitors the battery voltage, current, and temperature to ensure safe and efficient performance while protecting the battery from overcharging, over-discharging, and overcurrent conditions. The LCD display provides real-time information such as vehicle speed, battery state of charge, and trip distance, enabling the rider to monitor the operating status of the bike. The hydraulic telescopic front forks and rear hydraulic shock absorbers absorb road irregularities, improving ride comfort and stability, while the Brembo hydraulic disc brakes provide reliable braking performance for safe deceleration and stopping. The integrated operation of these electrical and mechanical systems ensures efficient, safe, and environmentally friendly transportation for two riders.

Cost Estimation

The estimated manufacturing cost of the proposed electric bike is based on the prices of commercially available components and standard fabrication charges. The values presented are approximate and may vary depending on the supplier, component quality, and manufacturing location.

- Frame and chassis fabrication (AISI 1018 Mild Steel): Approximately PKR 35,000

- BLDC Hub Motor (2 kW): Approximately PKR 70,000

- Lithium-Ion Battery Pack (60 V, 30 Ah): Approximately PKR 95,000

- BLDC Motor Controller: Approximately PKR 12,000

- Battery Management System (BMS): Approximately PKR 8,000

- LCD Display: Approximately PKR 5,000

- Hall-Effect Throttle: Approximately PKR 2,000

- Brembo Hydraulic Disc Brake Set: Approximately PKR 28,000

- Hydraulic Telescopic Front Suspension: Approximately PKR 18,000

- Rear Hydraulic Shock Absorbers: Approximately PKR 15,000

- 17-inch Alloy Wheels: Approximately PKR 18,000

- Tubeless Tires: Approximately PKR 12,000

- Handlebar Assembly: Approximately PKR 5,000

- Dual Rider Seat: Approximately PKR 6,000

- LED Lighting System (Headlight, Tail Light & Indicators): Approximately PKR 6,000

- Smart Battery Charger: Approximately PKR 10,000

- Electrical Wiring, Connectors and Switches: Approximately PKR 5,000

- Bearings, Fasteners and Miscellaneous Hardware: Approximately PKR 8,000

- Painting and Surface Finishing: Approximately PKR 8,000

- Assembly and Fabrication Labour: Approximately PKR 20,000

Based on the above estimates, the total manufacturing cost of the proposed electric bike is approximately PKR 386,000. If the design is manufactured on a commercial scale, the cost can be reduced through bulk procurement, optimized production processes, and economies of scale.

Limitations of the Proposed Model

The proposed electric bike has been designed using engineering calculations, CAD modeling, and component selection; however, certain limitations exist due to the scope of the project.

- The project is limited to the design and analysis stage, and a physical prototype has not been fabricated.

- The performance values presented in the report are predicted from engineering calculations and manufacturer specifications rather than experimental testing.

- The estimated driving range and maximum speed may vary depending on rider weight, road conditions, terrain, weather, and riding style.

- Structural strength and durability have been evaluated analytically; experimental fatigue and impact testing have not been performed.

- Battery life and charging performance are based on the selected battery specifications and may differ under practical operating conditions.

- The proposed design does not include advanced features such as regenerative braking, GPS navigation, anti-theft tracking, mobile application connectivity, or fast-charging capability.

- The cost estimation is approximate and may change due to fluctuations in market prices, manufacturing methods, and component availability.

- Prototype fabrication and comprehensive road testing are required before the design can be considered for commercial production or mass manufacturing.

These limitations indicate that the proposed model demonstrates the technical feasibility of the electric bike design, while future work should focus on prototype development, experimental validation, and performance optimization before commercialization.

Conclusion

This project presented the design and analysis of an Electric Bike as an efficient and environmentally friendly alternative to conventional fuel-powered motorcycles. The proposed design incorporates a 2 kW BLDC motor, 60 V lithium-ion battery, hydraulic suspension, Brembo hydraulic disc brakes, and an LCD display to achieve a maximum speed of approximately 50 km/h while safely carrying two adult riders.

The selection of components and engineering calculations indicate that the proposed design is capable of providing good performance, reliability, rider safety, and energy efficiency for urban transportation. Although the project is limited to the design stage and has not been physically fabricated or tested, the analytical results demonstrate the technical feasibility of the proposed electric bike. Future work should focus on prototype fabrication, experimental testing, and performance optimization to validate the design and prepare it for practical implementation and commercial production.

AutoCAD Model:

----------Orthographic Views----------

(Side View)

(Front View)

(Top View)

(Cutting Section Line)

(Section View)

----------Assembly Views----------

(Battery)

(Brembo Brakes)

(Wheels - Tires, Rims and Axles)

(BLDC Motor)

(Suspensions)

REFERENCES:

- https://www.brembo.com/en/solutions/for-original-equipement-bike/icons/stylema-caliper

- https://www.kyb.co.jp/en/products/motorcycle.html

- https://mechtex.com/blog/working-of-bldc-motor

- https://www.iqytechnicalcollege.com/BAE%20685-Electric%20Vehicle%20Technology.pdf

- https://www.sciencedirect.com/science/article/pii/S2352152X26008510?utm_source=chatgpt.com